GAUGE 1 MODEL RAILWAY ASSOCIATION – STANDARD DIMENSIONS

1 – RAIL PROFILES:

Bullhead rail profile        Flat Bottom rail profile

# – TRACK SPACING British outline – Centre to Centre

Straight running lines        -115mm minimum

Curved running lines, under 3650mm radius,

- 120mm minimum

Curved running lines, under 2450mm radius,

-125mm minimum

Straight sidings                 -120mm minimum

Supplement to G1MRA standards – suggested track spacing for non-British stock.

As a result of input from members who run nonBritish outline stock, we suggest the following centre line spacing as a starting point:

Parallel track    - 135mm tolerance -0/+10mm

Curves (tight)   - 148mm tolerance -0/+12mm

Curves             - 140mm tolerance -0/+15mm

Trackbed area, clear of obstructions (to avoid footsteps etc) 68mm from centre line on curves 60mm on straights.

Stock Gauge – depending on stock but 232mm high will just allow American “Double-Stack Container cars” through. Allow extra side clearance in curved tunnels etc.

# – ELECTRIC PICK-UP

Conductor rail –

Height above running rail (top) – 3.0mm

Centre from inside edge of running rail – 14.00mm

Overhead line  - Proposed Standards

Height of contact wire above rail – 160mm tolerance -10/+20

Maximum offset from centre line of track 12.5mm

Stud Contact

Skate dimensions. Allow 3.2mm rise at ends

|  | Standard |  | Fine |  |
| --- | --- | --- | --- | --- |
| Skate width | 20 | mm | 12.5 | mm |
| Studs: |  |  |  |  |
| Maximum spacing | 114 | mm | 114 |  |
| Maximum height above rails | 1.6 | mm | 1.6 | mm |
| Maximum rise between adjacent studs | 1.6 | mm | 1.6 | mm |
| Maximum offset from centre – line of track at turnouts | 8 | mm | 4.8 | mm |

# – BUFFERS AND COUPLINGS

# Buffers    Height

above rail - 35mm

Centres - 57mm

# Couplings – links

1.2 diameter

5 – STOCK GAUGE British Prototypes

All dimensions in mm                        RG 17/04/09